Chemeca 2026 and Hazards Australasia

28 – 30 September, Melbourne, Australia

A hybrid machine learning model for bioprocess scale-up

Kenneth S.Y. Nga,b,c, Tizita Horningc,d, Alekhya Penumarthic,d, Eleanor Cumminsc,d, Irene Bakerc,d, Yih Yean Leec,d, Sally L. Grasa,b,c

a Department of Chemical Engineering, The University of Melbourne, Parkville, VIC 3010, Australia.

b The Bio21 Molecular Science and Biotechnology Institute, The University of Melbourne, Parkville, Vic 3010, Australia.

c ARC Digital Bioprocess Development Hub, The University of Melbourne, Parkville, VIC 3010, Australia.

d CSL Innovation, 655 Elizabeth Street, Melbourne, VIC 3000, Australia.

Email: sinnyn@unimelb.edu.au

ABSTRACT

Monoclonal antibodies (mAbs) are a class of biopharmaceuticals that are strongly in-demand, due to their high specificity and versatility. To meet this growing demand, there is a need for quicker process transfer from developmental stages to commercial production (i.e. scaling up). Complex interrelationships between process conditions, bioreactor physics, mass and energy transfer and culture behaviour, however, make process scale-up challenging. To achieve quicker process transfer, more accurate and efficient scaling methods are necessary.

Here, data-driven models could be helpful for their ability to process complex multivariate datasets, such as bioreactor cell culture data, though they have limited capacity for extrapolation and typically require considerable amounts of diverse data, whereas new processes targeted for scaling typically only contain data for a limited number of experiments under fairly uniform conditions. These shortcomings can be addressed by hybrid models, which incorporate mechanistic components and have been demonstrated to outperform data-driven counterparts while needing less data. Hybrid models have been explored in bioprocessing, mainly in the areas of process development and process control but not for bioreactor scaling.

In this work, a hybrid machine learning model was developed to scale Chinese Hamster Ovary (CHO) cell cultures from bench to pilot scale. By supplementing historical data consisting of bioreactor runs spanning multiple scales, with a small number of bench scale runs of a new process, the hybrid model was able to accurately predict culture growth behaviour of the new process at pilot scale. Factors affecting model behaviour and performance were also investigated.

KEY WORDS

Hybrid modeling, mammalian cell culture, bioprocessing, scaling, machine learning

BIOGRAPHY

Dr. Kenneth Ng is a Technical Research Support Officer in the Department of Chemical Engineering at the University of Melbourne. His research focuses on improving bioprocess development and manufacturing through machine learning approaches. He is a part of the ARC Digital Bioprocessing Development Hub, a collaboration between industry and researchers for the digital transformation of the Australian biopharmaceutical manufacturing sector.

Conference Program

Please indicate which conference program your abstract relates to:

Hazards Australasia

Chemeca