Understanding Ion Transport in Cascading Nanoslit Systems Embedded in Graphene Membranes

(Jefferson) Zhe Liu A

A Department of Mechanical Engineering, The University of Melbourne, Parkville, Australia;

Ion transport in nanoconfinement differs from that in bulk and has been extensively researched across scientific and engineering disciplines. However, investigation of the transport properties of ions confined in nanoporous carbon is generally difficult because of the stochastic nature and distribution of multiscale complex and imperfect pore structures within the bulk material. We demonstrate a combined approach of experiment and simulation to describe the structure of complex, layered graphene-based membranes, which allows their use as a unique porous platform to gain unprecedented insights into nanoconfined transport phenomena across the entire sub–10-nm scales. By correlation of experimental results with simulation of concentration-driven ion diffusion through the cascading layered graphene structure with sub–10-nm tunable interlayer spacing, we are able to construct a robust, representative structural model that allows the establishment of a quantitative relationship among the nanoconfined ion transport properties in relation to the complex nanoporous structure of the layered membrane. This correlation reveals the remarkable effect of the structural imperfections of the membranes on ion transport and particularly the scaling behaviours of both diffusive and electrokinetic ion transport in graphene-based cascading nanochannels as a function of channel size from 10 nm down to subnanometer.

Besides, we exploit the tunable nanoconfinement in layered graphene-based membranes, to show that sub-2 nm confined ion diffusion can be strongly modulated by the surface potential induced electric double layer (EDL). Depending on the potential sign, the combination and concentration of ion pairs, diffusion rates can be reversibly modulated and anomalously enhanced by 4~7 times within 0.5 volts, across a salt concentration gradient up to seawater salinity. Modelling suggests that this anomalously enhanced diffusion is related to the strong ion-ion correlations under severe nanoconfinement, and cannot be explained by conventional theoretical predictions.

Our analysis shows that the range of ion transport effects previously observed in simple one-dimensional nanofluidic systems will translate themselves into bulk, complex nanoslit porous systems in a very different manner, and the complex cascading porous circuities can enable new transport phenomena that are unattainable in simple fluidic systems.

References

- Cheng, C., Jiang, G., Garvey, C. J., Wang, Y., Simon, G. P., Liu, J. Z., and Li, D. (2016). Ion transport in complex layered graphene-based membranes with tuneable interlayer spacing, Science Advances 2, e1501272.

- Cheng, C., Jiang, G., Simon, G. P., Liu, J. Z., and Li, D. (2018) Low-voltage electrostatic modulation of ion diffusion through layered graphene-based nanoporous membranes, Nature Nanotechnology 13, 685