AT estimation: trimming the V̇CO2-V̇O2 relationship to a biphasic response

Simon Green1,2, Matt Goldsmith1, Belinda Cochrane3,4

1School of Health Sciences, Western Sydney University, NSW, Australia. 2Department of Cardiology, Campbelltown Hospital, NSW, Australia. 3Department of Respiratory, Campbelltown Hospital, NSW, Australia. 4School of Medicine, Western Sydney University, NSW, Australia

Introduction/Aim: Estimation of the anaerobic threshold (AT) from V̇CO2-V̇O2 relationship is complicated by evidence that this relationship often consists of three thresholds. Corroboration of additional thresholds with the expired ventilation rate (V̇e) might be useful to refining visual-based methods of estimating AT. Method: 10 healthy participants completed a maximum ramp protocol (10 W.min-1) on a cycle ergometer with ventilation and respiratory gas fractions measured breath-by-breath. A four-phase function was used to identify three thresholds in V̇CO2-V̇O2, V̇e-V̇CO2 and V̇e-V̇O2 responses, namely the initial adaptive threshold (IAT), AT and respiratory compensation threshold (RCT). AT estimates from the fitting of the four-phase function were compared with AT estimates from a new procedure which incorporated V̇e measurements and V̇CO2-V̇O2 data trimmed of responses below and above IAT and RCT, respectively. Results: IAT was not significantly different between V̇CO2-V̇O2 (0.95 ± 0.28 L.min-1), V̇e-V̇CO2 (0.88 ± 0.49 L.min-1) and V̇e-V̇O2 (0.97 ± 0.43 L.min-1), although these estimates were notcorrelated (r = 0.18 to 0.52, P > 0.05). AT was not significantly different between V̇CO2-V̇O2 (1.68 ± 0.55 L.min-1), V̇e-V̇CO2 (1.80 ± 0.41 L.min-1) and V̇e-V̇O2 (1.69 ± 0.51 L.min-1) but estimates were significantly correlated (r > 0.82, P < 0.05). RCT was not significantly different between V̇CO2-V̇O2 (2.27 ± 0.35 L.min-1), V̇e-V̇CO2 (2.40 ± 0.41 L.min-1) and V̇e-V̇O2 (2.34 ± 0.51 L.min-1) and estimates were significantly correlated (r > 0.87, P < 0.05). AT estimated from fitting trimmed V̇CO2-V̇O2 data to a biphasic function yielded AT estimates (1.67 ± 0.52 L.min-1) similar to estimates from fitting all V̇CO2-V̇O2 data to a 4-phase function (absolute differences = 0.05 ± 0.06 L.min-1). Conclusion: These data provide support for an improved AT estimation procedure which eliminates data below IAT and above RCT but which can still make use of a biphasic visual aid. Key Words: CPET, anaerobic threshold estimation, initial adaptive threshold, respiratory compensation threshold Grant Support: NA