First Results from the Time Resolved Opacity Spectrometer on the NIF

Y. P. Opachich1, P.M. Kozlowski2, B. Golick1, J. G. Buscho1, A. Carpenter1, P.B. Cho1, J. W. Corbin1, E. C. Dutra3, J. Emig1, B. T. Funsten1, A. M. Garafalo1, J. Heinmiller3, M. Hohenberger1, H. M. Johns2, C. Kenstler3, R. Knight3, K. Larsen3, B. Lechleiter1, D. Max3, S. B. Morioka1, K. J. Moy4, P. R. Nyholm1, N. Palmer1, A. E. Peterson3, R. B. Petre1, R. S. Posadas1, A. M. Sharp1, W. Stolte3, J. A. Tabimina1, V. Tran3, S. D. Trent3, M. S. Wallace3, T. Archuleta2, A. Zubelewicz2, T. S. Perry2, T. J. Urbatsch2 and R. F. Heeter1

1Lawrence Livermore National Laboratory, Livermore, California 94550, USA

2Los Alamos National Laboratory, Los Alamos, New Mexico 87545, USA

3Nevada National Security Sites, Livermore, California 94550, USA

4Nevada National Security Sites, Santa Barbara, California 93117, USA

Corresponding Author Email: opachich1@llnl.gov

Abstract. Radiative opacity plays a central role in determining stellar structure, with iron contributing significantly to the solar opacity, particularly near the radiation–convection boundary. Current atomic models of opacity remain a major source of uncertainty, contributing to discrepancies between standard solar models and helioseismic observations. Agreement with models could be attained if key opacity values were 10-50% larger, motivating high-precision experimental benchmarks at stellar interior conditions. The Opacity-on-NIF campaign measures iron opacity via x-ray transmission through samples heated in a hohlraum. Previous measurements using the time-integrated OpSpec spectrometer have been limited to ~150 eV and electron densities of ~10²² cm⁻³. Higher temperatures are achievable, but measurements were precluded by strong background emission. To address this limitation, a new time-resolved spectrometer, OpSpecTR, has been developed. OpSpecTR employs an elliptically bent crystal and grazing-incidence x-ray mirror to achieve high spectral purity while minimizing background through a small crossover slit. Time-resolved spectra in the 1–2 keV range are recorded on 3 fast-gated hCMOS detectors, simultaneously measuring transmitted, backlighter, and self-emission signals required for opacity extraction. Nanosecond-scale gating isolates the backlighter pulse and reduces background contributions by up to 80%. Spectral resolving power is also improved ~2x from the time-integrated instrument. This approach enables significantly improved fidelity in opacity measurements under high-temperature conditions, advancing experimental constraints on stellar opacity models.

This work was performed under the auspices of the U.S. Department of Energy by Lawrence Livermore National Laboratory under Contract DE-AC52-07NA27344. LLNL-ABS-xxxxx.