PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

Minimum material strength of binary asteroid Didymos-Dimorphos from the perspective of structural stability

Yun Zhang(1), Patrick Michel(1), Derek C. Richardson(2), Harrison F. Agrusa(2), Kleomenis Tsiganis(3), Olivier S. Barnouin(4), and Özgür Karatekin(5)

(1) Université Côte d'Azur, Observatoire de la Côte d'Azur (OCA), CNRS, Laboratoire Lagrange, 06304 Nice, France, yun.zhang@oca.eu, michelp@oca.eu;

(2) Department of Astronomy, University of Maryland, College Park, MD 20740-2421, United States, dcr@umd.edu, hagrusa@astro.umd.edu;

(3) Department of Physics, Aristotle University of Thessaloniki, Thessaloniki, Greece, tsiganis@auth.gr;

(4)The Johns Hopkins University, Applied Physics Laboratory, United States, Olivier.Barnouin@jhuapl.edu;

(5) Royal Observatory of Belgium, Avenue Circulaire 3, 1180 Uccle, Belgium, ozgur.karatekin@observatory.be.

Keywords: Binary asteroid system, cohesive rubble pile, YORP spin-up effect, tidal effect, discrete element modeling.

##### ABSTRACT

The binary asteroid 65803 Didymos-Dimorphos system is the target of the first asteroid deflection test (NASA Double Asteroid Redirection Test, DART) and of the first rendezvous mission with a binary asteroid system and internal structure investigation (ESA Hera). The cohesive strengths of both the fast-spinning primary (Didymos) and the secondary (Dimorphos) are key factors that could significantly affect the impact outcome and stability of this system [1, 2]. To support the preparation and data interpretation of these missions and gain a better understanding of the formation and evolution of this system, we investigate the structural stability and cohesive strengths of both Didymos and Dimorphos according to current observational constraints.

Based on the assumption that the Didymos’ current fast rotation is driven by the YORP spin-up effect, we explicitly simulated the spin-up process of Didymos from a slow spin state to its current spin state using a high-efficiency soft-sphere-discrete-element-model code, pkdgrav [1, 3]. Several Didymos rubble-pile models with a shape matching the radar shape model [4] were constructed by using different particle arrangements and size distributions. Dimorphos is modeled as either a point mass or a rubble-pile ellipsoid. By assessing the creep stability of the rubble pile structures at the end of the simulations, we can obtain the minimum required material strength to keep the shape of each rubble-pile body.

Our previous studies [5, 6] have shown that in the absence of the secondary, Didymos requires a small amount of bulk cohesion (~10 Pa) to maintain its structural stability at its current fast spin state. In this study, we include the secondary and explore the mutual tidal effect on the structural stability and evolution of this system.

The preliminary results show that, due to the tidal effect of Dimorphos, there is an increment in the minimum required bulk cohesion on the order of 0.1–1 Pa for the considered Didymos rubble pile models. The exact amount depends on the rubble-pile configuration and the associated failure mode. For the case where the rubble pile tends to fail internally, a larger increment is needed; for the case where the rubble pile tends to fail through surface shedding, the tidal effect of Dimorphos on the minimum required cohesion of Didymos is negligible. Our numerical experiments also show that the rubble-pile Dimorphos can be stable with a friction angle of ~40 deg without cohesion (in the case of synchronous rotation).

As a next step, we will investigate in depth the tidal evolution of this system via full-two-rubble-pile modeling based on the material parameters derived from the current study. Comparisons with measurements by the DART and Hera missions will allow constraining the mechanical properties and history of this binary asteroid.

Acknowledgments: Y. Z. acknowledges funding from the Université Côte d’Azur “Individual grants for young researchers” program of IDEX JEDI. D.C.R., O.S.B., and H.F.A. acknowledge support from the NASA Planetary Defense Program and the DART mission, under NASA Contract #NNN06AA01C to Johns Hopkins University/Applied Physics Laboratory. This project has received funding from the European Union’s Horizon 2020 research and innovation program under grant agreement No. 870377.

References:

[1] Zhang, Y., Richardson, D.C., Barnouin, O.S., Michel, P., Schwartz, S.R., Ballouz, R.L. Rotational failure of rubble-pile bodies: Influences of shear and cohesive strengths.  The Astrophysical Journal 857, 15 (2018).

[2] Raducan, S.D., and Jutzi, M. Global scale deformations caused by the DART impact: Insights to the collisional evolution of small asteroids. 52nd LPSC abstract #1900 (2021).

[3] Schwartz, S.R., Richardson, D.C., Michel, P. An implementation of the soft-sphere discrete element method in a high-performance parallel gravity tree-code. Granular Matter 14, 363–380 (2012).

[4] Naidu, S.P., Benner, L.A.M., Brozovic, M., Nolan, M., Ostro, S., Margot, J., Giorgini, J., Hirabayashi, T., Scheeres, D., Pravec, P., Scheirich, P., Magri, C., Jao, J. Radar observations and a physical model of binary near-Earth asteroid 65803 Didymos, target of the DART mission. Icarus 348, 113777 (2020).

[5] Zhang, Y., Richardson, D.C., Barnouin, O.S., Maurel, C., Michel, P., Schwartz, S.R., Ballouz, R.L., Benner, L.A.M., Naidu, S.P., Li, J. Creep stability of the proposed aida mission target 65803 Didymos: I. discrete cohesionless granular physics model. Icarus 294, 98–123 (2017).

[6] Zhang, Y., Michel, P., Richardson, D.C., Barnouin, O.S., Agrusa H.F., Tsiganis K. Creep stability of the DART/Hera mission target 65803 Didymos: II. the role of cohesion. Submitted to Icarus (2021).

**************************************

Comments:

Oral preferred.

This contribution could be included into the DART-Hera missions session.