PDC2021
Vienna, Austria
Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission 
You may visit https://iaaspace.org/pdc
(please choose one box to be checked)
(you may also add a general comment - see end of the page)

 FORMCHECKBOX 
 Key International and Political Developments
 FORMCHECKBOX 
 Advancements and Progress in NEO Discovery
 FORMCHECKBOX 
 NEO Characterization Results
 FORMCHECKBOX 
 Deflection and Disruption Models & Testing
 FORMCHECKBOX 
 Mission & Campaign Designs
 FORMCHECKBOX 
 Impact Consequences
 FORMCHECKBOX 
 Disaster Response
 FORMCHECKBOX 
 Decision to Act
 FORMCHECKBOX 
 Public Education & Communication
Mass Driver Asteroid Deflection with Area-of-Effect Softbots
Jay McMahon(1) and Daniel Brack(2)
(1) University of Colorado Boulder, XXXXX, XXXXX, Jay.Mcmahon@colorado.edu
(2) Lockheed Martin Space, XXXXX, XXXXX, daniel.n.brack@lmco.com
Keywords: Maximum of five keywords separated by comma
ABSTRACT

Many different methods have been proposed for deflecting potentially hazardous objects, however they generally fall into two categories: fast methods (i.e. nuclear or kinetic deflection) or long-lead methods (e.g. ion-beam, gravity tractor, mass drivers). In this talk, we focus on the concept of mass driver deflection enabled by a new spacecraft concept called Area-of-Effect Softbots (AoES) which have been developed under support from the NASA Innovative Advanced Concepts (NIAC) program.  
AoES are soft-robotic spacecraft with large, actuated surface areas as pictured in Fig. 1 which allow for unique mobility and surface anchoring through contact electroadhesive forces. The ability to anchor regardless of the asteroid subsurface structure enables to possibility of digging and launching material from the surface of a small NEA as pictured. By controlling when we launch the material from the surface, and by repeating this process for a period of time, we can produce measurable changes to an asteroid’s orbit, and thus could conceivably be used as an asteroid deflection option given enough warning time. 

[image: image1.png]Controlled
Landing

; Material Extraction





Figure 1 - Overview illustration of the AoES concept.
In this talk, we will present an overview of the AoES design and simulation results to illustrate the baseline capabilities of such a system for asteroid deflection. 
**************************************

Comments:

This work has been largely published in: Brack, Daniel N., and Jay W. McMahon. "Active Mass Ejection for Asteroid Manipulation and Deflection." Journal of Spacecraft and Rockets 57, no. 3 (2020): 413-433.

