PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

REGOLITH CONSTRAINTS FOR NEAR-EARTH ASTEROID 2021 PDC

Annika Gustafsson(1), Eric MacLennan(2), Nicholas Moskovitz(3) , and David E. Trilling(4)

- Department of Astronomy and Planetary Science, Northern Arizona University, Flagstaff, AZ 86001, USA, ag765@nau.edu

- Department of Physics, PO Box 64, 00014 University of Helsinki, Finland, eric.maclennan@helsinki.fi

- Lowell Observatory, 1400 West Mars Hill Road, Flagstaff, AZ 86001, USA, nmosko@lowell.edu

- Department of Astronomy and Planetary Science, Northern Arizona University, Flagstaff, AZ 86001, USA, david.trilling@nau.edu

Keywords: physical properties, photometry, thermal modeling, radiative transfer modeling

##### ABSTRACT

Asteroid regolith grain size is diagnostic of several poorly understood geophysical processes and has direct planetary defense applications. For example, it is well known that a rubble pile size barrier exists around asteroid diameters of 200-300m [1]. Small bodies with diameters below this size can rotate quickly with periods less than 2.2 hours, however, large objects generally do not rotate that quickly. This rotation rate barrier is attributed to a transition from large, gravity dominated objects to small, strength dominated ones [2]. Yet, it is unclear how this transition affects surface properties. Thermal (e.g. [3]) and in-situ (e.g. [4]) data suggest that object diameter influences surface grain size -- large bodies have fine surface grains and small bodies have coarse grains. However, rotation rate [5] and size-independent processes like thermal fracturing [6] and tidal interactions with planets [7] may be equally important in affecting surface properties.

For planetary defense risk assessment, quantifying the nature of the regolith and near-subsurface material of potential impactors can inform early predictions of the momentum transfer from a kinetic impactor [8].

We will present an overview of Hapke radiative transfer and thermal modeling tools that we have adopted to constrain near-Earth asteroid surface regolith grain sizes. These can be used for hazard assessment of close-approaching near-Earth asteroids and for better understanding the surface characteristics of objects above and below the size barrier. In this work, we focus on the application of these tools with respect to characterizing surface properties of near-Earth asteroid 2021 PDC. Specifically, we will apply our modeling and data analysis tools to synthetic observations of 2021 PDC from the 4.3-m Lowell Discovery Telescope and NASA’s Infrared Telescope Facility.

We have adopted a probabilistic implementation of Hapke radiative transfer modeling [9] on unresolved asteroids to constrain surface regolith grain size. By collecting high signal-to-noise (S/N > 50) photometry of objects going through opposition, we can fit the data with our Hapke photometric model to constrain regolith grain size.

Using multi-epoch thermal measurements that are collected at opposition and across a wide range of solar phase angles, we can estimate thermal inertia and, from it, the surface grain size following methods by [10] and [11]. A reasonable estimate for the rotation period is needed for this approach, but shape and spin properties are treated as unknown free-parameters when fitting to the observations. Detailed knowledge of the lightcurve period and amplitude can be used to constrain the range of these free parameters.

We will compare the thermal-derived grain size estimates to those constrained by our implementation of the Hapke photometric model to better understand the surface properties of the body across multiple wavelengths. For example, a bare-rock thermal inertia value and coarse Hapke grain size may suggest an intact monolith, as opposed to a rubble pile comprised of various-sized boulders and fine surface grains.

References: [1] Warner, B. D., Harris, A. W., Pravec, P. 2009. The asteroid lightcurve database, in: Icarus 202, 134–146. [2] Harris, A. W. 1996. Effects of Shape and Spin on the Tidal Disruption of P/Shoemaker-Levy 9, in: Earth Moon and Planets 72, 113–117. [3] Delbo', M., dell'Oro, A., Harris, A. W., Mottola, S., Mueller, M. 2007. Thermal inertia of near-Earth asteroids and implications for the magnitude of the Yarkovsky effect, in: Icarus 190, 236–249 [4] Dellagiustina, D. N. and 58 colleagues 2019. Properties of rubble-pile asteroid (101955) Bennu from OSIRIS-REx imaging and thermal analysis, in: Nature Astronomy 3, 341–351. [5] Harris, A. W., Drube, L. 2016. Thermal Tomography of Asteroid Surface Structure, in: The Astrophysical Journal 832. [6] Delbo, M. and 8 colleagues 2014. Thermal fatigue as the origin of regolith on small asteroids, in: Nature 508, 233–236. [7] Binzel, R. P. and 9 colleagues 2010. Earth encounters as the origin of fresh surfaces on near-Earth asteroids, in: Nature 463, 331–334. [8] Jutzi, M. 2015. SPH calculations of asteroid disruptions: The role of pressure dependent failure models, in: Planetary and Space Science 107, 3–9. [9] Hapke, B. 1981. Bidirectional reflectance spectroscopy. I – Theory, in: Journal of Geophysical Research 86, 3039–3054. [10] MacLennan, E. M., Emery, J. P. 2019. Thermophysical Modeling of Asteroid Surfaces Using Ellipsoid Shape Models, in: The Astronomical Journal 157.  [11] Gundlach, B., Blum, J. 2013. A new method to determine the grain size of planetary regolith, in: Icarus 223, 479–492.

**************************************

Comments:

(Oral)