PDC2021

Vienna, Austria

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

ESA’S ACTIVITIES IN PLANETARY DEFENCE

Detlef Koschny(1,5,6), Juan L. Cano(1,2), Ramona Cennamo(1,3), Luca Conversi(1,10), Ernesto Doelling(7), Laura Faggioli(1,3), Angelo Foglietta(1,4), Michael Frühauf(6), Gianpiero Di Girolamo(7), Rainer Kresken(1,8), Udo Kugel(7), Marco Micheli(1,3), Richard Moissl(1,5), Pablo Ramirez Moreta(1,9), Dario Oliviero(1,2), Elisabeta Petrescu(1,7), Regina Rudawska(1,3), Gunter Sessler(7)

(1)ESA NEO Coordination Centre, Via Galileo Galilei, 00044 Frascati (RM), Italy, neocc@ssa.esa.int

(2)Elecnor Deimos, Via Giuseppe Verdi, 6, 28060 San Pietro Mosezzo (NO), Italy

(3)RHEA Systems, Via di Grotte Portella, 6/8, 00044 Frascati (RM), Italy

(4)Alia Space Systems, Via Lunati, 10, 00044 Frascati (RM), Italy

(5)ESA ESTEC, Keplerlaan 1, 2201 AZ Noordwijk, The Netherlands

(6) LRT / TU Munich, Boltzmannstraße 15, Garching bei München 85748, Germany

(7)ESA ESOC, Robert-Bosch-Straße 5, 64293 Darmstadt, Germany

(8)CGI Deutschland B.V. & Ko. KG, Rheinstrasse 95, 64295 Darmstadt, Germany

(9)GMV, Isaac Newton 11, Tres Cantos, 28760 Madrid, Spain

(10)ESA ESRIN, Largo Galileo Galilei 1, 00044 Frascati (RM), Italy

Keywords: Planetary Defence Office, observations, information provision, mitigation

##### ABSTRACT

The European Space Agency (ESA) has been active in the field of planetary defence since January 2009. As part of its Space Situational Awareness programme, ESA has set up an ‘NEO segment’. In 2018, the ‘Planetary Defence Office (PDO)’ was established. Since January 2020, it is part of ESA’s Space Safety Programme.

The mission statement of the Space Safety programme is the

“… protection of our planet, humanity and assets in space and on Earth from dangers originating in Space” (PB-SSA 2018(24)).

For the PDO:

(a) Be aware of the situation of natural objects in space;

(b) predict possible impacts and their consequences, inform relevant parties;

(c) prepare for risk mitigation, by technological developments and on political level.

The PDO team is structured in three ‘pillars’:

(1) Observations;

(2) Information provision;

(3) Mitigation.

Most activities are performed by personnel physically located at our NEO Coordination Centre, at ESRIN, ESA’s location in Frascati, Italy. All software/hardware is supported by the Space Safety ground segment team, see Di Girolamo et al. (this conference).

‘Observations’: The PDO has set up a network of follow-up observers and telescopes, mainly with telescopes of aperture 40 to 200 cm. In a collaboration with the European Southern Observatory, we have regular access to the 8.2 m Very Large Telescope. For more information, see Conversi et al., Micheli et al. (this conference).

ESA is developing a survey telescope, called ‘Flyeye’ telescope with OHB Italia as prime contractor. Integration of the cameras (16 in total) is in process. In parallel, the optical train is being tested. The production of the site infrastructure for the final telescope location (Mt. Mufara, Sicily, Italy) has started.

The PDO is also studying the possibility for Europe to contribute to radar observations.

‘Information provision’: In 2020, the PDO has established an independent capability to determine orbits and predict near-Earth object impacts. The software is an evolution of the NEODyS system, initially developed by the Univ. Pisa and operated by the company SpaceDyS, both Italian. Objects with a computed impact probability greater than 0 for the next 100 years will be posted on the so-called ‘risk page’. Following the agreement with the International Astronomical Union, for predicted impact probabilities larger than a Palermo Scale value of −2, the computations are double-checked with JPL/NASA. For more information and additional services, see Cano et al., Frühauf et al. (this conference).

‘Mitigation’ covers a number of activities: ESA’s PDO is setting up interfaces to emergency response agencies in member countries, is actively involved in the International Asteroid Warning Network and the Space Mission Planning Advisory Group. To demonstrate the capabilities of asteroid deflection, ESA is building the Hera mission which will complement NASA’s DART mission impacting asteroid Dimorphos, part of a binary system. ESA is preparing two CubeSat missions: M-ARGO, studying a small asteroid; LUMIO, observing impact flashes on the lunar far side. The two CubeSat missions are currently in Phase A, with funding currently being secured for actual implementation.

References

Cano, L. et al. (2021): Recent evolutions in ESA’s NEOCC system. This conference.

Conversi, L. et al. (2021): ESA’s NEOCC observational network. This conference.

Frühauf, M. et al. (2021): Meerkat Asteroid Guard imminent impactor warning service of the European Space Agency. This conference.

Di Girolamo, G. et al. (2021): NEOCC DevOps-based operations. This conference.

Micheli, M. et al. (2021): Recent observational highlights from ESA’s NEO Coordination Centre. This conference.

**************************************