Li-N-H Speciation in Molten LiCl-KCl-CsCl Eutectic with Operando Gas Analysis

Pooja Dhami, Ian J. McPherson

Loughborough University

Loughborough, United Kingdom

p.p.dhami@lboro.ac.uk

Ammonia is one of the most produced chemicals globally, and its demand continues to rise to sustain economic and population growth. Currently, ammonia is predominantly synthesized via the Haber-Bosch process, which accounts for 1.8% of global CO₂ emissions and 2% of global energy consumption, necessitating the development of alternative synthesis methods that operate using milder conditions. This work investigates nitrogen reduction reaction in a molten chloride electrolyte and aims to further the understanding of reaction pathways for a more sustainable method of ammonia production, which is less energy intensive and doesn’t require the high pressures needed for conventional ammonia synthesis.

The molten ternary eutectic (LiCl:KCl:CsCl, 57.5:13.3:29.2 mol%) electrolyte is used in this work enhances reaction kinetics through its high ionic conductivity and a wide electrochemical window,1 allowing for the analysis of the redox active species over a broad potential range while also suppressing the undesirable hydrogen evolution reaction and providing a stabilizing environment for nitrogen containing intermediates. These properties make this a suitable system to study the electrochemically driven Li-N-H speciation.

A custom-designed cell was assembled, comprising a 3D-printed alumina crucible, porous Ni foam gas diffusion electrodes and an Ag/AgCl reference electrode. Electrochemical analysis  was carried out and signature redox peaks observed in the cyclic voltammograms were attributed to the electrochemically driven Li-N-H speciation.2,3 A key aspect of this work is the integration of operando gas analysis. The gas outlet of the furnace setup was connected to an online infrared (IR) gas cell, enabling real-time, continuous detection of the gaseous products. This electrochemical approach with operando analytical detection provides insights on reaction pathways is expected to provide insight on the Li-H-N speciation in molten chloride melts, laying down the groundwork for studies on electrochemical ammonia synthesis.

1. T. Murakami, T. Nohira, Y. H. Ogata, and Y. Ito, Electrochem. Solid-State Lett., 8, E1 (2004).

2. I. J. McPherson et al., Angewandte Chemie International Edition, 58, 17433–17441 (2019).

3. S. Feng, W. Gao, J. Guo, H. Cao, and P. Chen, ACS Energy Lett., 8, 1567–1574 (2023).