Functionalized Ni single-atom catalysts on UiO-66 MOFs for efficient photo-electro-catalytic remediation of pharmaceutical contaminants and CO₂ reduction

Shivaraju Harikarahalli Puttaiah1 , Divya Vinoda 2, Tenzin Thinley 3

1 Department of Environmental Sciences, JSS Academy of Higher Education and Research, Mysuru-570015, India

2Moshivin Academy of Science and Technology (MAST), Shihashi Edu. Private Limited, Harikaranahalli Village, Tumkur District, 572215, Karnataka, India

3Center for Water, Food and Energy, GREENS TRUST, Harikaranahalli Village, Tumkur District, 572215, Karnataka, India

*Corresponding author:

Shivaraju Harikaranahalli Puttaiah (shivarajuenvi@gmail.com), Ph: +91-8277102057.

Abstract: The persistence of pharmaceutical contaminants and the rapid increase in anthropogenic CO₂ emissions necessitate advanced multifunctional remediation strategies. Using a solvothermal-reduction method, Ni single-atom catalysts anchored on UiO-66 (Ni-SAC/UiO-66) were synthesised in this work and characterised systematically using HR-TEM-HAADF STEM + EDS, PXRD, FTIR, XPS, BET, PL, and UV-Vis analyses. It was confirmed that fundamentally dispersed Ni could be incorporated into the UiO-66 framework, offering a large number of active sites, enhanced charge separation and enhanced capture of visible light by suppressing electron-hole recombination. The hybrid catalyst performed better than each of the components to achieve over 91% ampicillin degradation in 120 minutes under visible light irradiation. Studies using reactive oxygen species trapping showed that the primary oxidative species leading to the degradation had been photogenerated holes (h⁺). Additionally, through facilitating the photoreduction of CO₂ into hydrocarbons, the material demonstrated strong potential for sustainable chemical conversion. Its continuous applicability has been proven by the photocatalyst's exceptional durability and reusability over several cycles. Ni-SAC/UiO-66 is a promising multifunctional photocatalyst for sustainable applications because it can simultaneously combat environmental pollution through solar-driven degradation and enable resource valorisation through CO₂ conversion.

Keywords: Single atom catalyst, metal organic framework, antibiotic degradation, CO2 photoreduction, sustainability.