The Mechanical Stability Limit of Microporous Carbons Defining The Transition From Reversible Charging-Induced Deformation To Mechanical Fatigue

Rabea Saleh Rabea Bin Sowad, Krzysztof Fic

Poznan University of Technology

ul. Jacka Rychlewskiego 1, 61-131 Poznan, Poland
rabea.binsowad@put.poznan.pl

Microporous carbon electrodes in aqueous electrical double-layer capacitors experience significant mechanical stresses during ion electrosorption. Here, operando electrochemical dilatometry was employed to quantify the real-time dimensional response of microporous carbons during charging and identify the structural compliance limit of the carbon framework. Differential electromechanical analysis revealed a distinct transition from reversible elastic expansion to irreversible mechanical fatigue. Across various aqueous electrolytes, permanent structural degradation emerged beyond a characteristic charge-storage threshold corresponding to the exhaustion of pore compliance. Increased ion confinement and desolvation-induced stresses beyond this limit accelerated structural degradation, with heavily hydrated ions imposing the greatest mechanical burden. These findings establish the structural compliance limit as a physically grounded criterion defining the boundary between elastic charging and mechanical fatigue, highlighting the critical role of mechanical endurance in determining the operational lifetime of aqueous electrochemical capacitors.