The Felts, They Are A-Changing: Three-Electrode Dynamic Electrochemical Impedance Spectroscopy as an Operando Electrode Ageing Monitoring Tool for Redox Flow Batteries

Wojciech Bącalski, Eliza Hałas, Paweł Ślepski, Joanna Krakowiak

Faculty of Chemistry, Gdańsk University of Technology

Narutowicza 11/12, 80-233 Gdańsk, POLAND

wojciech.bacalski@pg.edu.pl

Vanadium redox flow batteries (VRFB) experience performance losses driven by electrode ageing [1], yet the underlying mechanisms remain poorly explored under operational conditions [2]. This presentation shows a novel application of three-electrode dynamic electrochemical impedance spectroscopy (DEIS) to monitor kinetic and transport processes throughout complete charge and discharge cycles of a flow cell from three simultaneous perspectives of full-cell and respective half-cells. Four different electrode states have been examined: pristine, thermally activated, electrochemically aged, and chemically aged. Complementary EIS measurements at selected states of charge were performed to highlight the substantial differences between spectra recorded under load and at open-circuit conditions. The results reveal that the impedance response of the full cell is dominated by processes occurring at the negative electrode, and that ageing leads to increased charge-transfer resistance and enhanced SoC-dependent variation.

The study demonstrates-for the first time under realistic VRFB cycling conditions- that DEIS provides unique diagnostic capabilities, enabling mechanistic insights into electrode ageing that are inaccessible to conventional impedance approaches.

[1]	X. Z. Yuan et al., “A review of all ‐ vanadium redox flow battery durability : Degradation mechanisms and mitigation strategies,” no. April, pp. 6599–6638, 2019, doi: 10.1002/er.4607.

[2]	J. Krakowiak, W. Bącalski, G. Lentka, P. Peljo, and P. Ślepski, “Three modes of electrochemical impedance spectroscopy measurements performed on vanadium redox flow battery,” Sustain. Mater. Technol., vol. 40, no. April, 2024, doi: 10.1016/j.susmat.2024.e00957.