Decoding complex interfacial behaviors by combining advanced atomistic simulations and x-ray spectroscopy

Piotr M. Kowalski

AffiliationInstitute of Energy Technologies: Theory and Simulation of Energy Materials (IET-3), Forschungszentrum Jülich GmbH, Germany

e-mail address: p.kowalski@fz-juelich.de

Powerful supercomputing resources enable large scale simulations of complex electrochemical interfaces at molecular-level. We will discuss a few cases of successful application of atomistic simulations conducted to support experimental effort aiming at understanding the performance of selected electrocatalysts and materials for batteries. In particular, we will demonstrate power of computational effort in interpreting XANES and EXAFS measurements on redox-active transition metal elements. These efforts enabled us to get insights into, for instance, influence of spin states of active elements on efficiency of OER reaction and stability of selected candidate materials for battery cathodes [1]. While discussing specific scientific cases, we will outline difficulties and challenges faced by atomistic simulation effort and demonstrate that educative selection of accurate electronic structure computational methodology is essential to produce reliable results. We will briefly discuss advantage of the advanced DFT+U computational scheme we develop and use, in which special attention is given to make the method parameter free and to realistically describe targeted d orbitals [2,3].

[1] He et al., Nat Commun 14, 3498 (2023)

[2] Kowalski et al. Front. Energy Res. 9, 653542 9, 653542 (2021)

[3] Warda et al. J. Chem. Theory Comput. 22, 1016–1029 (2026)