Phase Engineering of Nanomaterials (PEN)

Hua Zhang

Department of Chemistry, Hong Kong Institute for Clean Energy (HKICE), Hong Kong Branch of National Precious Metals Material Engineering Research Center (NPMM), City University of Hong Kong, Hong Kong, China

Shenzhen Research Institute, City University of Hong Kong, Shenzhen, 518057, China

hua.zhang@cityu.edu.hk

In this talk, I will summarize the recent research on phase engineering of nanomaterials (PEN) in my group, particularly focusing on the rational design and synthesis of novel nanomaterials with unconventional phases for various promising applications. For example, by using wet-chemical methods, for the first time, we have successfully prepared novel Au nanostructures (e.g., the hexagonal-close packed (hcp) 2H-Au nanosheets, 4H-Au nanoribbons, and 4H/fcc and fcc/2H/fcc heterophase Au nanorods), epitaxially grown metal nanostructures on the aforementioned unconventional Au nanostructures and 2H-Pd nanoparticles, and amorphous/crystalline heterophase Pd, PdCu, Rh and Rh alloy nanosheets. By using gas-solid reactions, metastable 1T'-phase group VI transition metal dichalcogenides (TMDs), e.g., WS2, WSe2, MoS2, MoSe2, WS2xSe2(1-x) and MoS2xSe2(1-x), have been prepared. Impressively, the 1T'-MoS2-supported single-atomically dispersed Pt (s-Pt) atoms with Pt loading up to 10 wt% exhibit superior performance in hydrogen evolution reaction. Importantly, 1T'-TMD monolayers can be stabilized on 4H-Au nanowires, which have been used for ultrasensitive SERS detection. Moreover, the salt-assisted 2H-to-1T' phase transformation of TMDs have been achieved, and the phase transformation of TMDs during our developed electrochemical Li-intercalation process has been observed. Impressively, the lithiation-induced amorphization of Pd3P2S8 has been achieved. Currently, my group focuses on the investigation of (crystal) phase-dependent physicochemical properties, functions and applications in catalysis, (opto-)electronic devices, clean energy, chemical and biosensors, surface enhanced Raman scattering, water remediation, photothermal therapy, etc., which we believe are quite unique and very important not only in fundamental studies, but also in future practical applications. Importantly, the concepts of phase engineering of nanomaterials (PEN), crystal-phase heterostructures, and heterophase materials are proposed.