Better than just overpotential and chronopotentiometry? Investigating activity and stability metrics for multi-objective exploration of water oxidation catalysts

Jan Goran T. Tomacruz1,2, Soren B. Scott1,2

1Department of Chemistry, University of Copenhagen, Copenhagen, Denmark

2Sino-Danish College, University of Chinese Academy of Sciences, Beijing, China

jgt@chem.ku.dk

The ongoing development of electrocatalysts for water electrolyzer anodes is a multi-objective optimization problem, as various data-driven theoretical1 and experimental2 approaches are being done to discover new compositions and improve performance. Catalysts are typically evaluated by activity and stability, often measured respectively using overpotential and chronopotentiometry or chronoamperometry time, but these metrics alone can lead to oversimplified and erroneous interpretations3,4. Here, we propose two objectives: turnover frequency at a set potential5, and price-normalized stability number6. We use oxygen evolution reaction (OER) in alkaline media as a starting point, since OER is a key kinetic bottleneck and alkaline electrolysis is the most mature electrolyzer technology7. We also construct methodologies to extract these objectives or calculate lower-bound estimates from a wide range of data reporting conventions and applicable assumptions in the literature. For example, all atoms in a material are assumed active if active sites were not quantified, which would result to a lower-bound turnover frequency. Similarly, all active metals are assumed to dissolve at the end of chronopotentiometry or chronoamperometry measurements if exact dissolution was not measured, resulting in a lower-bound stability number. A Pareto front is also plotted on catalysts that are Pareto optimal8, that is if one catalyst is not outperformed by another on either activity or stability. Although this work can function as a reference for further studies on catalysts on this Pareto front, a stronger point to emphasize is that more accurate activity and stability techniques must be conducted for more reliable comparisons and multi-objective optimizations.

References:

- Zhang, C.; Lu, R.; Sun, Q.; Mao, Y.; Söhnel, T.; Zhao, Y.; Truhlar, D. G.; Wang, Z. Finding the Pareto Front for High-Entropy-Alloy Catalysts. Chemical Science 2026, 17 (9), 4744–4752. https://doi.org/10.1039/d5sc06100h.

- Jiang, X.; Wang, Y.; Jia, B.; Qu, X.; Qin, M. Using Machine Learning to Predict Oxygen Evolution Activity for Transition Metal Hydroxide Electrocatalysts. ACS Applied Materials & Interfaces 2022, 14 (36), 41141–41148. https://doi.org/10.1021/acsami.2c13435.

- Kibsgaard, J.; Chorkendorff, I. Considerations for the Scaling-up of Water Splitting Catalysts. Nature Energy 2019, 4 (6), 430–433. https://doi.org/10.1038/s41560-019-0407-1.

- Wei, C.; Wang, Z.; Otani, K.; Hochfilzer, D.; Zhang, K.; Nielsen, R.; Chorkendorff, I.; Kibsgaard, J. Benchmarking Electrocatalyst Stability for Acidic Oxygen Evolution Reaction: The Crucial Role of Dissolved Ion Concentration. ACS Catalysis 2023, 13 (21), 14058–14069. https://doi.org/10.1021/acscatal.3c03257.

- Kozuch, S.; Martin, J. M. L. “Turning Over” Definitions in Catalytic Cycles. ACS Catalysis 2012, 2 (12), 2787–2794. https://doi.org/10.1021/cs3005264.

- Geiger, S.; Kasian, O.; Ledendecker, M.; Pizzutilo, E.; Mingers, A. M.; Fu, W. T.; Diaz-Morales, O.; Li, Z.; Oellers, T.; Fruchter, L.; Ludwig, A.; Mayrhofer, K. J. J.; Koper, M. T. M.; Cherevko, S. The Stability Number as a Metric for Electrocatalyst Stability Benchmarking. Nature Catalysis 2018, 1 (7), 508–515. https://doi.org/10.1038/s41929-018-0085-6.

- International Energy Agency; The Future of Hydrogen 2019, 32. https://www.iea.org/reports/the-future-of-hydrogen.

- Campigotto, P.; Passerini, A.; Battiti, R. Active Learning of Pareto Fronts. IEEE Transactions on Neural Networks and Learning Systems 2014, 25 (3), 506–519. https://doi.org/10.1109/tnnls.2013.2275918.