Detection of 1-Methyl-4-N-nitrosopiperazine (MNP) in antitubercular drugs using MIP based sensor

Kacper Bieńkowski,1 Maciej Cieplak,1 Krzysztof Stolarczyk,2 Agnieszka Lis-Cieplak,3 Elżbieta Stolarczyk,3 Francis D'Souza,4 and Piyush S. Sharma1

1 Institute of Physical Chemistry, Polish Academy of Sciences, Kasprzaka 44/52, 01-224, Warszawa, Poland

2 Faculty of Chemistry, University of Warsaw, Pasteura 1, 02-093 Warszawa, Poland

3 Spectrometric Methods Department, National Medicines Institute, Chełmska 30/34, 00-725 Warsaw, Poland

4 Department of Chemistry, University of North Texas, Denton, 1155, Union Circle, #305070, TX 76203-5017, USA

mcieplak@ichf.edu.pl

1-Methyl-4-N-nitrosopiperazine (MNP) constitutes toxic contamination of rifampicin, an anti-tuberculotic drug. Therefore, methods of nitrosamines determination are needed. Especially in field of pharmaceutical products quality control [1,2]. Nitrosamine detection methods that utilize chromatographic techniques with mass spectrometry detection are labor-intensive, time-consuming, expensive, and require the use of complex equipment and appropriately trained personnel. Therefore, fast and inexpensive electrochemical assay based on electrode coated with molecularly imprinted polymer (MIP) was proposed. For that purpose, MIP-film was deposited on the electrode using electropolymerization. 1-Methyl-4-N-nitrosopiperazine was extracted from the polymer, and the current response of the prepared electrode coated with the MIP layer was measured using differential pulse voltammetry. The devised sensor allowed for determination of MNP in pM concertation range with LOD=12.9 pM. Selectivity of the sensor was sufficiently high to determine MNP in samples of pharmaceutical products containing rifampicin, ethambutol, isoniazid and pyrazinamide. The obtained results were cross-validated using LC-MS/MS method.

Figure 1. Structural formula of MNP, rifampicin, ethambutol, isoniazid and pyrazinamide

Acknowledgments

The financial support from the National Science Centre, Poland Project No. 2022/47/B/ST5/02337 (to M.C.) is acknowledged.

References

- Witkowska, A. B.; Zezula, M.; Glice, M.; Stolarczyk, E. U. New Aspects of the Methodology of N-Nitrosamines Determination. Przemysl Chemiczny 2021, 100 (3), 230–239. https://doi.org/10.15199/62.2021.3.3.

- Witkowska, A. B.; Wołczyńska, A.; Lis-Cieplak, A.; Stolarczyk, E. U. Development and Validation of LC-MS/MS Method for the Determination of 1-Methyl-4-Nitrosopiperazine (MNP) in Multicomponent Products with Rifampicin—Analytical Challenges and Degradation Studies. Molecules 2023, 28 (21). https://doi.org/10.3390/molecules28217405.