# Automating Building Code Interpretation Using NLP and Transfer Learning for Enhanced Code Compliance

Sherief Ali, Markus König

Chair of Computing in Engineering, Ruhr-Universität Bochum, Germany

ABSTRACT:

Building codes are essential for ensuring safety, quality, and regulatory compliance in construction projects. Currently, extracting information from regulatory texts is manual, time-consuming, and prone to errors, particularly because it involves interpreting natural language. This paper focuses on leveraging deep learning to train a model capable of predicting essential entities from regulatory texts, thereby enabling the interpretation and understanding of the natural language used in regulations. We fine-tuned a pre-trained BERT (Bidirectional Encoder Representations from Transformers) model to perform entity recognition for CODE-ACCORD dataset, which contains English and Finnish regulatory clauses annotated with BIO (Begin, Inside, Outside) tags across four essential categories: Object, Property, Quality, and Value. The trained model achieved high precision in entity recognition, effectively generalizing predictions to previously unseen regulatory clauses.. This paper contributes primarily by automating the semantic interpretation of regulatory texts, thereby establishing a foundation for more efficient and accurate automated code compliance checking processes.

KEYWORDS:

Code Compliance, Deep learning, Regulations, Text interpretation, Transformer, Entity recognition, BERT

# 1. INTRODUCTION

The Architecture, Engineering, and Construction (AEC) industry is characterized by continually pursuing innovation. Regulatory documents play a role, providing the framework within which construction projects operate from start to end. Regulations are critical for ensuring that construction projects meet safety, environmental, and efficiency standards. Traditionally, extracting information from regulatory documents and formulating rules has been a typically manual task (Eastman et al., 2009). However, this manual process is time-consuming, labor-intensive, and often inconsistent interpretations (Borrmann et al., 2018).

Automatic Code Compliance (ACC) has emerged as a crucial research focus aiming to verify building models against regulatory requirements. Currently, methods for verifying compliance range from manual and semi-automated tools to advanced techniques involving rule-based systems and advanced machine-learning techniques (Fuchs et al., 2022). Traditional rule-based methods depend heavily on human interpretation to translate the regulatory clauses written in natural language into rules, which leads to errors such as misinterpretation of legal terms and inconsistencies in rule formulation (Eastman et al., 2009). On the other hand, machine learning, particularly deep learning methods, reduces reliance on manual rule interpretation and encoding. These approaches leverage extensive datasets of regulatory texts to learn patterns and extract relevant compliance rules with minimal human intervention. Additionally, the machine learning approach emphasizes understanding the semantics of regulations by leveraging large datasets to learn patterns, interpret semantics, and extract actionable compliance information (Alnuzha & Bloch, 2025).

This paper leverages deep learning to train a model for automatically extracting essential entities from regulatory texts, addressing the challenges associated with manual interpretation, which is typically time-consuming and error-prone. Therefore, we utilize a pre-trained transformer model, BERT, to automatically interpret regulatory texts by training it on the CODE-ACCORD dataset, which includes annotated building regulations in English and Finnish. The framework is designed to train on sentences annotated using BIO tags across four specific categories: Object, Property, Quality, and Value. Consequently, the model predicts these entities in unseen regulatory clauses. The network has effectively interpreted the natural language of regulations and accurately predicted entities in unseen regulatory clauses. The results demonstrate strong performance in entity prediction, achieving a precision score of 0.98. This high precision highlights the model's capability to accurately extract relevant entities from regulatory texts, thus reducing errors and improving text interpretation.

# 2. RELATED WORK

ACC is explored in various research studies, employing a range of deep learning models. Different approaches are designed to interpret the natural language of building codes and to capture the semantics of sentences. Natural Language Processing (NLP) techniques are crucial in these efforts, allowing systems to process and understand regulatory texts. For instance, Named Entity Recognition (NER), which identifies and classifies entities within text, and Part-of-Speech (POS) Tagging, which assigns parts of speech to words to better understand their grammatical roles (Manning et al., 2014). (Zhang & El-Gohary, 2022) proposed a deep learning-based approach to generate intelligent building codes. They introduced a semantic representation called the Multi-Form Semantic (MFS) requirement hierarchy. This hierarchy organizes requirements hierarchically into units and their interrelations, with each unit further defined by Semantic Information Elements (SIEs) such as subject, compliance attributes, and quantitative details. An intelligent code is defined as natural-language requirements linked to their hierarchies. The authors trained a Recurrent Neural Network (RNN) on 7,500 annotated sentences. They tested it on 600 manually created sentences to generate coherent intelligent building-code requirements by feeding requirement units into the model. Other studies focus on the relations in the text. For instance, (Wang & El-Gohary, 2023) proposed a relation extraction method using deep learning to identify relations from unstructured text in construction safety requirements. The authors evaluated their model using two deep learning architectures: Convolutional Neural Network (CNN)  and RNN, and stated that the CNN-based model captured features more precisely. (Xue et al., 2024) addressed the common challenge of limited training data by presenting a deep learning and NLP-based question-answering (QA) framework for building codes, consisting of two main components: a "retriever" and a "reader." The retrievers used sparse and dense features, while the readers utilized transformer-based language models fine-tuned to a building code dataset. While (Yang & Zhang, 2024) integrated Generative Pretrained Transformers (GPT) models with prompt engineering to combine the precision of rule-based systems with the flexibility of machine learning. Using a prompt-based approach, the GPT model can interpret and translate the natural language of building codes into machine-readable formats.

Accordingly, this paper focuses on interpreting regulatory text and predicting entities in new clauses. This enhances the understanding of sentence semantics and entity prediction for unseen regulatory clauses.

# 3. RESEARCH METHODOLOGY

This section outlines the comprehensive approach to interpreting the natural language of regulatory clauses by training entities on a pre-trained BERT model. Figure 1 illustrates the conceptual framework used in this study. Section 3.1 outlines the dataset structure, annotation, and description of BIO tags (Object, Property, Quality, Value) within CODE-ACCORD dataset. Accordingly, these annotated data undergo preprocessing as detailed in Section 3.2. Subsequently, these data are used for training and fine-tuning the pre-trained BERT model, which is discussed in detail in Section 3.3. The trained model is tested on unseen regulatory clauses to predict entities, enabling their semantic interpretation and classification.

Figure 1: Framework of the study

3.1 Data Preparation

A key challenge in ACC is the lack of publicly available datasets, which could significantly accelerate the development and testing of new methodologies and techniques. As highlighted by (Amor & Dimyadi, 2021), there is no publicly available regulatory dataset that can be used to evaluate the quality of ACC systems. (Hettiarachchi et al., 2024) addressed this issue by introducing the CODE-ACCORD corpus, which contains 862 clauses extracted from building regulations in both England and Finland. The authors manually labeled and annotated a total of 4,297 entities in the CODE-ACCORD corpus.. The dataset is split into training and testing, each containing sentences and their corresponding tags. These sentences are formatted in a structured manner, with both the raw text extracted and associated tags listed in CSV format (Hettiarachchi et al., 2023).

The sentences in the dataset (Hettiarachchi et al., 2023) are annotated using the Begin-Inside-Outside (BIO) tagging scheme (Ramshaw & Marcus, 1995), which is widely used for labeling sequences in tasks such as NER. The BIO tags are applied to every word in the clauses to denote whether a word is the beginning of a specific entity (B-), inside the entity (I-), or outside of any entity (O). For instance, in the phrase "Load-bearing wall", the entity "Load-bearing wall" is annotated as "B-tag I-tag I-tag" because all the words describe the same noun, 'wall', with the description starting from 'load' and ending at 'wall'. However, 'O' denotes any word that does not contribute meaningful information to the sentence

This BIO scheme ensures that the boundaries of multi-word entities, such as legal terms, are marked, allowing for precise entity recognition. The authors (Hettiarachchi et al., 2024) established a set of specific entity categories to guide the tagging of sentences in the CODE-ACCORD corpus, ensuring that every term within the clauses is assigned to one of these predefined categories. This systematic approach helps maintain consistency in labeling and supports the accurate classification of regulatory terms. These entity categories, described in Table 1, include key elements that are central to defining compliance criteria in building regulations.

Table 1: Description of entity categories from the CODE-ACCORD dataset (Hettiarachchi et al., 2023)

| Named entity | Description |
| --- | --- |
| Object | An ontological concept that represents a thing that is subject to a particular requirement (e.g. window, fire door) |
| Property | Property of an object (e.g., width, height) |
| Quality | Unlike properties, qualities are not always quantifiable (e.g., horizontal, self-closing) |
| Value | Numerical measurement or defined quantity associated with a property (e.g. 1,500 millimeters, five percent) |

Based on the entity categories described in Table 1, the sentences in the dataset are annotated according to these specific labels (Hettiarachchi et al., 2024), ensuring that each term is appropriately tagged. This annotation process is illustrated in Table 2, where each word in the sentence is assigned a corresponding BIO tag that reflects its role within the regulatory clause.

Table 2: Examples of annotated clauses in the dataset

Code clauses

The area of windows, roof windows, rooflights and doors should not exceed 25 of the total floor area of the dwelling.

The (O) area (B-property) of (O) windows (B-object), (O) roof (B-object) windows (I-object), (O) rooflights (B-object) and (O) doors (B-object) should (O) not (O) exceed (O) 25 (B-value) of (O) the (O) total (B-property) floor (I-property) area (I-property) of (O) the (O) dwelling (B-object). (O)

On stairs and ramps, a handrail must be installed for the entire length and on both sides of the structure.

On (O) stairs (B-object) and (O) ramps (B-object), (O) a (O) handrail (B-object) must (O) be (O) installed (O) for (O) the (O) entire (B-quality) length (I-quality) and (O) on (O) both (B-quality) sides (I-quality) of (O) the (O) structure (B-object). (O)

3.2 Data Preprocessing

The sentences are tokenized using the BERT tokenizer, where words are split into subwords as necessary. The tokenization process ensures that each subword is assigned the same label as its corresponding word, a crucial step to preserve label alignment with the tokenized sequence (Devlin et al., 2019). Special tokens [CLS] and [SEP] are added to the beginning and end of the tokenized sequence to mark the sentence boundaries, following BERT's input format. Sentences that exceed the fixed maximum length of 512,  are truncated, while shorter sentences are padded with [PAD] tokens to ensure uniform input size (Vaswani et al., 2017). Alongside the tokens, attention masks are generated to indicate which tokens are real and which are padding, and the BIO tags are converted into numerical label IDs using a predefined label-to-ID mapping.

3.3 Model Development

The model architecture begins with a BERT base uncased pre-trained language model. The BERT base uncased is trained on a corpus of unsupervised English data (HuggingFace, 2018). This foundational layer provides a strong starting point for fine-tuning specific tasks, leading to improved prediction precision. To train the model, the dataset elaborated in Section 3.1 consists of a training set comprising 690 sentences; thus, we have allocated 20% % of this data for validation purposes. Additionally, the test dataset comprises 174 sentences, which are evaluated to assess the model's performance.

# 4. RESULTS AND ANALYSIS

4.1 Model Evaluation

To evaluate the predictions from the model training, precision, recall, and F1 are utilized as key metrics to assess the network's performance. Precision quantifies the accuracy of the positive predictions made by the model, indicating the proportion of true positive (TP) instances among all predicted positives. While false negatives (FN) denote the false predictions by the network. The F1 serves as a harmonic mean of precision and recall. Scores approaching 1 or 100% are an indication of optimal performance, whereas values nearing 0 signify inadequate effectiveness.

The network has achieved results as detailed in Table 3, having a precision of 0.989, a recall of 0.988, and an F1-score of 0.989. These metrics indicate that the model is highly effective in accurately identifying the entities within the regulatory clauses.

Table 3: Performance of the network

| Evaluation metrics | Performance |
| --- | --- |
| Precision | 0.989 |
| Recall | 0.988 |
| F1-score | 0.989 |

Furthermore, the accuracy of the model was assessed for each BIO tag, as illustrated in Table 4. The results have shown that the B-Object tag achieved an accuracy of 0.997, while the I-Object tag followed closely with an accuracy of 0.998. Notably, the B-Quality tag also demonstrated a remarkable accuracy of 0.996, highlighting the model's capability to recognize quality-related entities. Additionally, the O (outside) tag maintained an accuracy of 0.996.

Table 4: Predictions of annotated entities with BIO tags

| BIO tag | Accuracy | Validated accuracy |
| --- | --- | --- |
| B-Object | 0.997 | 0.61 |
| I-Object | 0.998 | 0.60 |
| B-Property | 0.995 | 0.72 |
| I-Property | 0.997 | 0.63 |
| B-Quality | 0.996 | 0.80 |
| I-Quality | 0.997 | 0.90 |
| B-Value | 1.0 | 0.93 |
| I-Value | 1.0 | 0.82 |
| O(Outside) | 0.996 | 0.99 |

There are specific BIO tags, such as B-Object and I-Property, that exhibit lower accuracy values. This variation highlights the presence of class imbalance within the dataset, which can impact the model's ability to accurately predict less frequent tags.

4.2 Model Predictions

Based on the model evaluation discussed in Section 4.1, the trained network is used to generate predictions on the test dataset. The predictions focus on identifying entities within regulatory sentences using the BIO tagging scheme. Table 5 presents regulatory sentences from the test dataset, along with their ground truth annotations and the predicted tags assigned by the model. It provides a comparison between the expected labels and the model's predictions, highlighting areas where the classification was accurate as well as instances of misclassification.

Table 5: Predictions on new clauses from the test dataset

| Sentence 1 | Large roof surfaces must be divided into parts of no more than 2,400 square meter |
| --- | --- |
| Ground truth | Large (B-quality) roof (B-object) surfaces (B- property) must (O) be (O) divided (B-quality) into (I-quality) parts (I-quality) of (I-quality) no (I- quality) more (I-quality) than (I-quality) 2,400 (I- quality) square (I-quality) meters (I-quality). (O) |
| Predicted tags | Large (B-quality) roof (B-object) surfaces (I-object) must (O) be (O) divided (B-quality) into (I-quality) parts (I-quality) of (O) no (O) more (O) than (O) 2,400 (B-value) square (I-quality) meters (I- quality).(O) |
| Sentence 2 | However, when the snow load can occur, the mini- mum value of the load s should be at least 0.5 kN/m2. |
| Ground truth | However (O), (O) when (O) the (O) snow (B-object) load (I-object) can (O) occur (B-quality), (O) the (O) minimum (B-property) value (I-property) of (O) the (O) load (B-object) s (O) should (O) be (O) at (O) least (O) 0.5 (B-value) kN/m2 (I-value).(O) |
| Predicted tags | However (O), (O) when (O) the (O) snow (B-object) load (I-object) can (O) occur (O), (O) the (O) minimum (B-property) value (I-property) of (O) the (O) load (B-property) s (I-property) should (O) be (O) at (O) least (O) 0.5 (B-value) kN/m2 (I-value). (O) |

Notably, the predicted tags exhibit certain misclassifications. For instance, there is an over-prediction of the "O" tag, where words that should have been categorized under entity labels were instead classified as outside the relevant categories. Additionally, the model incorrectly predicted "2,400" as a B-value, while in the ground truth, it is labeled as I-quality. This misclassification likely occurred because numerical values generally belong to the B-value category in the dataset. As a result, the model learned this pattern and mistakenly assigned "2,400" the B-value tag instead of recognizing its actual role in the given context.

# 5. CONCLUSION

In this paper, we developed a framework to semantically interpret regulatory text, to have a full understanding of the natural language of sentences. We have trained a network built upon a BERT base uncased model that is fine-tuned on entities labeled with BIO tags from the CODE-ACCORD dataset, achieving high predictive accuracy for various tags essential for regulatory interpretation. The trained model is tested on new regulatory clauses to predict the entity categories for unseen clauses. The model has shown strong performance in detecting entities within these clauses, indicating that the sentences have been successfully interpreted semantically.

This study has several limitations. One key constraint is the limited size of the dataset, which contains a relatively small number of annotated clauses. Additionally, although the model achieved high overall precision, certain entity categories exhibited lower prediction accuracy. These inconsistencies suggest that class imbalance within the dataset affects the model’s ability to generalize across all entity types.

In our future work, we plan to train the model on a more diverse set of regulatory clauses sourced from various building standards to enhance its generalizability and thoroughly evaluate its predictions. This will help improve the accuracy of both text interpretation and entity extraction. We also aim to demonstrate the complete workflow from entity prediction in regulatory clauses to compliance checking against BIM models. A key focus will be leveraging advanced language models to map the predicted entities to their corresponding IFC elements. Building on these developments, our ultimate goal is to develop a fully automated code compliance framework. For this to be effectively implemented, it is critical to adopt standardized, machine-readable data formats that support accurate interpretation, interoperability, and integration within digital workflows. Industry Foundation Classes (IFC) serve as a widely accepted standard for representing building information in a structured, neutral format suitable for automated rule checking (Clemen & Gruendig, 2006). Furthermore, Information Delivery Specifications (IDS) enhance this capability by defining explicit model exchange requirements in a computer-interpretable format (buildingSMART, 2024). Mapping regulatory clauses to IDS that align with the IFC schema will enable efficient and reliable compliance checking, streamlining the code compliance process.

# ACKNOWLEDGEMENTS

This research was funded by the mFUND research programme of the Federal Ministry for Digital and Transport (BMDV) (funding code: 01F2253B).

# REFERENCES

Alnuzha, M. & Bloch, T. (2025) The role of machine learning in automated code checking – a systematic literature review. Journal of Information Technology in Construction, 30, 22–44. Available from: https://doi.org/10.36680/j.itcon.2025.002.

Amor, R. & Dimyadi, J. (2021) The promise of automated compliance checking. Developments in the Built Environment, 5, 100039. Available from: https://doi.org/10.1016/j.dibe.2020.100039.

Borrmann, A., König, M., Koch, C. & Beetz, J. (2018) Building Information Modeling. Springer.

buildingSMART (2024) buildingSMART, 2024. Available from: https://​www.buildingsmart.org​/​.

Clemen, C. & Gruendig, L. (2006) The Industry Foundation Classes (IFC) -- Ready for Indoor Cadastre.

Devlin, J., Chang, M.-W., Lee, K. & Toutanova, K. (2019) BERT: Pre-training of Deep Bidirectional Transformers for Language Understanding.

Eastman, C., Lee, J., Jeong, Y. & Lee, J. (2009) Automatic rule-based checking of building designs. Automation in Construction, 18(8), 1011–1033. Available from: https://doi.org/10.1016/j.autcon.2009.07.002.

Fuchs, S., Witbrock, M., Dimyadi, J. & Amor, R. (2022) Neural Semantic Parsing of Building Regulations for Compliance Checking. IOP Conference Series: Earth and Environmental Science, 1101(9), 92022. Available from: https://doi.org/10.1088/1755-1315/1101/9/092022.

Hettiarachchi, H., Dridi, A., Gaber, M.M. & Parsafard, P. (2023) Accord-Project, 2023. Available from: https://​doi.org​/​10.5281/​zenodo.10210022. Available from: https://doi.org/10.5281/zenodo.10210022.

Hettiarachchi, H., Dridi, A., Gaber, M.M., Parsafard, P., Bocaneala, N. & Breitenfelder, K. et al. (2024) CODE-ACCORD: A Corpus of Building Regulatory Data for Rule Generation towards Automatic Compliance Checking.

HuggingFace (2018) BERT base uncased, 2018. Available from: https://​huggingface.co​/​google-​bert/​bert-​base-​uncased.

Manning, C., Surdeanu, M., Bauer, J., Finkel, J., Bethard, S. & McClosky, D. (2014) The Stanford CoreNLP Natural Language Processing Toolkit, 55–60. Available from: https://doi.org/10.3115/v1/P14-5010.

Ramshaw, L. & Marcus, M. (1995) Text Chunking using Transformation-Based Learning, In Third Workshop on Very Large Corpora.In Third Workshop on Very Large Corpora. Available from: https://​aclanthology.org​/​W95-​0107.pdf.

Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L. & Gomez, A.N. et al. (2017) Attention Is All You Need.

Wang, X. & El-Gohary, N. (2023) Deep learning-based relation extraction and knowledge graph-based representation of construction safety requirements. Automation in Construction, 147, 104696. Available from: https://doi.org/10.1016/j.autcon.2022.104696.

Xue, X., Zhang, J. & Chen, Y. (2024) Question-answering framework for building codes using fine-tuned and distilled pre-trained transformer models. Automation in Construction, 168, 105730. Available from: https://doi.org/10.1016/j.autcon.2024.105730.

Yang, F. & Zhang, J. (2024) Prompt-based automation of building code information transformation for compliance checking. Automation in Construction, 168, 105817. Available from: https://doi.org/10.1016/j.autcon.2024.105817.

Zhang, R. & El-Gohary, N. (2022) Natural language generation and deep learning for intelligent building codes. Advanced Engineering Informatics, 52, 101557. Available from: https://doi.org/10.1016/j.aei.2022.101557.